Gibbs phenomenom occurs when the analysis bandwidth is less than the

signal bandwidth.

Spectral leakage occurs when the analysis time is less than the

signal time.

On 2009-05-06 06:20:37 -0300, Rune Allnor <a...@tele.ntnu.no> said:

> On 6 Mai, 07:34, HyeeWang <hyeew...@gmail.com> wrote:

>> Gibbs phenomenon and spectral leakage.

>>

>> What is the relation between them?

>

> I'm inclined to say that there is no relation

> between them.

Others would say they are the same phenomena. Gibb's

is the result of truncating freuencies and looking

at the effects in time. Spectral leakage is the result

of truncating time and looking at the effects in frequency.

They are Fourier duals. The various schemes for dealing

with them are the same formulaes with one using t and the

other f. The formal names for these schmemes are various

weights. The informal names are often called windows in

less than formal settings. It can be confusing if you

only see one formally and the other informally.

> Gibbs phenomenon has to do with the FT representing

> a discontinuity by a continuous function. The Fourier

> basis vectors can represent continuous signals,

> but only approximate discontinuous signals.

The main attention in Gibb's is the poor approximation

at points of discontinuity. Just look at the convolution

of the original with the transform of the frequency

truncation window. This quickly shows that the relative

overshoot at a discontinuity will remain the same as

more terms are added, just the peak moves closer to the

break.

> Spectral leakage has to do with how the vectors that

> represent signals are being mapped onto the Fourier

> basis vectors. Unless the signal happens to match up

> with a small subset of the basis vectors, one needs

> contributions from all the basis vectors to represent

> the signal vector.

>

> Now, both Gibbs' phenomenon and spectral leakage

> have to do with signal representations in vector

> space, so *maybe* (just *maybe*) there might be

> some subtle similarites that might be seen as

> 'relations', deep, deep down.

Two sides of an isomorphism is not very subtle. And

it is isomorphism that is the bread and butter of

signal processing.

> But don't waste any time looking for that kind of

> stuff. At least wait till you have taken a few

> classes in real analysis, before you do.

You do not need measure theory, which is the usual

content of real analysis, when advanced (i.e. a second

course) calculus will do the job.

>> As a cause,which lead to which?

>

> There are no cause-effect relations between them.

>

> Spectral leakage occurs without Gibbs' phenomenon.

> Try to compute the N-pt DFT of a sinusoidal with

> frequency  Fs(2N-1)/2N*4. Spectrum leakage without

> Gibbs phenomenon.

>

> Gibbs' phenomenon exists without spectral leakage.

> Compute (analytically) the frequency response of an

> ideal square pulse, x(t) = 1, |t| < 1, 0 otherwise.

> Then IFT the spectrum to compute the time response

> from the frequency response. Gibbs' phenomenon

> is present.

>

> Rune