From the point of view of signal processing, the Gibbs phenomenon is the step response of a low-pass filter, and the oscillations are called ringing or ringing artifacts. Truncating the Fourier transform of a signal on the real line, or the Fourier series of a periodic signal (equivalently, a signal on the circle) corresponds to filtering out the higher frequencies by an ideal (brick-wall) low-pass/high-cut filter. This can be represented as convolution of the original signal with the impulse response of the filter (also known as the kernel), which is the sinc function. Thus the Gibbs phenomenon can be seen as the result of convolving a Heaviside step function (if periodicity is not required) or a square wave (if periodic) with a sinc function: the oscillations in the sinc function cause the ripples in the output.

The sine integral, exhibiting the Gibbs phenomenon for a step function on the real line.

In the case of convolving with a Heaviside step function, the resulting function is exactly the integral of the sinc function, the sine integral; for a square wave the description is not as simply stated. For the step function, the magnitude of the undershoot is thus exactly the integral of the (left) tail, integrating to the first negative zero: for the normalized sinc of unit sampling period, this is  The overshoot is accordingly of the same magnitude: the integral of the right tail, or, which amounts to the same thing, the difference between the integral from negative infinity to the first positive zero, minus 1 (the non-overshooting value).

The overshoot and undershoot can be understood thus: kernels are generally normalized to have integral 1, so they result in a mapping of constant functions to constant functions – otherwise they have gain. The value of a convolution at a point is a linear combination of the input signal, with coefficients (weights) the values of the kernel. If a kernel is non-negative, such as for a Gaussian kernel, then the value of the filtered signal will be aconvex combination of the input values (the coefficients (the kernel) integrate to 1, and are non-negative), and will thus fall between the minimum and maximum of the input signal – it will not undershoot or overshoot. If, on the other hand, the kernel assumes negative values, such as the sinc function, then the value of the filtered signal will instead be an affine combination of the input values, and may fall outside of the minimum and maximum of the input signal, resulting in undershoot and overshoot, as in the Gibbs phenomenon.

Taking a longer expansion – cutting at a higher frequency – corresponds in the frequency domain to widening the brick-wall, which in the time domain corresponds to narrowing the sinc function and increasing its height by the same factor, leaving the integrals between corresponding points unchanged. This is a general feature of the Fourier transform: widening in one domain corresponds to narrowing and increasing height in the other. This results in the oscillations in sinc being narrower and taller and, in the filtered function (after convolution), yields oscillations that are narrower and thus have lessarea, but does not reduce the magnitude: cutting off at any finite frequency results in a sinc function, however narrow, with the same tail integrals. This explains the persistence of the overshoot and undershoot.

good discussion of Ringing

http://en.wikipedia.org/wiki/Ringing_artifacts

### Ringing and ripple

Main articles: Ringing (signal) and Ripple (filters)

Frequency response of a 5th orderChebyshev filter, exhibiting ripple.

In signal processing and related fields, the general phenomenon of time domain oscillation is called ringing, while frequency domain oscillations are generally called ripple, though generally not "rippling".

A key source of ripple in digital signal processing is the use of window functions: if one takes an infinite impulse response (IIR) filter, such as the sinc filter, and windows it to make it have finite impulse response, as in thewindow design method, then the frequency response of the resulting filter is the convolution of the frequency response of the IIR filter with the frequency response of the window function. Notably, the frequency response of the rectangular filter is the sinc function (the rectangular function and the sinc function are Fourier dual to each other), and thus truncation of a filter in the time domain corresponds to multiplication by the rectangular filter, thus convolution by the sinc filter in the frequency domain, causing ripple. In symbols, the frequency response of  is  In particular, truncating the sinc function itself yields  in the time domain, and  in the frequency domain, so just as low-pass filtering (truncating in the frequency domain) causes ringing in the time domain, truncating in the time domain (windowing by a rectangular filter) causes ripple in the frequency domain.