Concepts to cover in a review paper about Spectral Analysis of EEG

Motivations for spectral analysis

-often more natural to analyze oscillatory signals like eeg in frequency domain

-problems:

--finite time: classical spectral analysis theory considers signals as either periodic or infinite in length

--randomness: infinitely many possible realizations of system output, but we only see 1

--nonstationarity: we are forced to analyze short segments because statistical properties change vs time

-theory of spectral estimation: gives way to estimate spectral content / frequency domain representation of the output of a random process (like eeg) based on finite data

-problem: finite data   bias and variance

Methods to reduce bias: tapering

Methods to reduce variance: smoothing

-first method to address this in nonparametric way while dealing with bias and variance at same time, “optimally”, is multitaper spectral estimation

Fig: EEG in  out = component parts

Spectra decomposition: EEG signals can be decomposed into sinusoidal oscillations: analysis and synthesis

Basic idea of spectral estimation : from a finite record of a stationary data sequence, estimate how the total power is distributed over frequency

Energy Spectral Density of Deterministic Signals

y(t); Assume finite energy -> then has DTFT Y(w); inverse DTFT; |Y(w)|^2 ESD; energy is same; these are different representations of same data (transformation, invertible)

Fig: show – random looking; show for wake, sleep

Power Spectral Density of Random Signals

EEG signals in practice are such that variation in the future cannot be known exactly; can only make probabilistic statements about variation (e.g. in the awake state, the average amplitude of the posterior rhythm will be 30uv). Math description of such signal: describe it as a random sequence, drawn from an ensemble of possible realizations, each of which has a certain probability to occur [Fig: bag of signals with same average characteristics, but random]

E[y(t)]=0, r(k)=E[y(t)y*(t-k)] autocovariance function – second order stationary sequences

Defns: (1) DTFT of autocov fxn; (2) limit for infinite N (data) of average value of squared DTFT of signal

- Note: to avoid aliasing, signal should be sampled at > Nyquist; signal must be bandlimited [show examples of signals with limited frequency content]

Nonparametric estimation methods:

Periodogram, correlogram: common methods, not good: high variance; explanation on p26 [1,2] & ch5

--interpretation in ch5 : power estimate based on single sample of filtered signal

Periodogram; is PSD based on non-infinite sample

Bias and variance; MSE = var(a)+|bias(a)|^2